WEEK:6

SAKSHAM RAO

Task Summary

The objective was to set up a fully functional OpenLane physical-design flow on native Ubuntu (without VirtualBox) and perform end-to-end RTL-to-GDS experiments using the SkyWater 130 nm PDK.

1. Environment Setup

| Component | Version / Source | Status |
| --- | --- | --- |
| OS | Ubuntu 24.04 LTS | ✅ |
| Python | 3.12.3 (venv) | ✅ |
| Docker | latest CE (from Docker repo) | ✅ |
| OpenLane | superstable branch (latest GitHub pull) | ✅ |
| OpenROAD / OpenSTA / Magic / KLayout | bundled via OpenLane | ✅ |
| PDK Root | $HOME/.volare/pdks | ✅ |
| SkyWater PDK | sky130A (built via make pdk) | ✅ — working |
| Example Designs | spm (available) / picorv32a (not bundled) | ⚠️ picorv32a missing |

2. Steps Performed

1. Cloned and built OpenLane

cd ~/tools

git clone https://github.com/The-OpenROAD-Project/OpenLane.git

cd OpenLane

python3.12 -m venv venv

source venv/bin/activate

pip install -r requirements.txt --break-system-packages

make

2. Installed SkyWater 130A PDK

export PDK_ROOT=~/.volare/pdks

make pdk

→ Successfully built sky130A under $PDK_ROOT/sky130A .

3. Veri�ed PDK

ls $PDK_ROOT/sky130A

→ Output shows libs.ref , libs.tech , sky130A_setup.tcl .

4. Validated �ow environment

make mount

cd /openlane/designs/spm

flow.tcl -design spm

→ OpenLane ran through synthesis and �oorplanning without errors.

5. Checked example repositories

• Con�rmed that the historical picorv32a example is no longer included in

current OpenLane.

• Investigating external sources for picorv32a RISC-V design to add manually.

3. Key Observations

• PEP 668 (environment restriction) required using --break-system-packages

during pip installs on Ubuntu 24.04.

•

ciel is a Python module, not an apt package; installing it through pip inside a

virtual environment �xes PDK build errors.

11/11/25, 23:49Physical design workshop guide

2 of 2

https://chatgpt.com/c/6913642d-9894-8320-bd93-6edcaef4b4f8

virtual environment �xes PDK build errors.

•

make pdk successfully built and enabled the SkyWater PDK through volare

manager.

• The PDK is operational and recognized by OpenLane; test �ow spm completes

through placement.

• The o�cial picorv32a example repository no longer exists under The-OpenROAD-

Project; alternative sources or custom design integration will be used.

4. Next Steps

1. Locate a public picorv32a design compatible with Sky130A (OpenLane ready) or

port from the archived OpenLane examples.

2. Perform the remaining �ow stages: CTS, routing, STA, DRC/LVS.

3. Document all command outputs and screenshots per lab in GitHub repository.

Evidence of Setup

- Proof Screenshot: